- Review the lecture slides on types of memory and provide a short answers to the following questions (using your own words):

  - What is ROM and what is its primary purpose ?

- Read-only Memory is a type of computer storage. It stores data that can only be read and the data cannot be loss even if the electric goes off

  - What is RAM and how is it different from ROM ?

- Random-access Memory is a type of computer storage. It stores the information which can be accessed quickly.

- It is different from ROM because when power goes off, the information that RAM store will be deleted.

  - What is the difference between static RAM and dynamics RAM ?

- The difference:

+ Static Ram: keep information until power removed, fast, larger 
area of silicon per byte, modest power requirement.

+ Dynamics RAM: keep information as long as the contents are 
refreshed frequently enough, smaller area of silicon per byte, low 
power requirement.

  - What type of memory is typically used in USB thumb drives ? Why shouldn’t we rely on this for critical data storage ?

- EEPROM (Flash memory) is used in USB. We shouldn’t rely on this for critical data storage because it can be easily affected by electricity.

- Consider a computer with 1GB RAM (1024 MB). Given memory addressing is for each byte, how many bits are needed to address all bytes in the system’s RAM ?

- We nees 2^30x8  to address all bytes in the system’s RAM.

- Give a brief description of the Von Neumann and Harvard computing architectures. What are the fundamental differences between the two and for what is is each designed to achieve ?

- The fundamental defferences:

+ Von Neumann architecture: Control bits and data bits share a common memory space hardware

+ Harvard architecture: instructions and data are kept separate.

- What is cache memory and what is its primary role ?

- Cache memory is used by CPU and store frequently accessed instructions/data in high-speed memory.

- Explain the concept of an interrupt, and list four common types.

- Interrupt is a type of signal from hardware that tell the CPU they want priority.

  - Polling is an alternative to interrupts ? Briefly explain polling and why it is not commonly used

- Polling is an alternative approach to interrupt. It is not commonly used because:

+ Can waste time checking hardware which is doing nothing.  
+ Doesn't take advantage of the stack.  
+ If one device freezes, this can make the  
entire computer unresponsive.

- Explain the general concept of a stack - how do they work, and what is their primary purpose.

- It is a row that can push data on and pop it out. A stack allows us to mothball/backup/hibernate a process/task at will on the receipt of an interrupt or code invocation.

  - How are stacks useful for handling interrupts ?

- Stack help device to continue the work after interrupts happen instead of finding where have the data gone after interrupts happen.

  - How are stacks useful in programming ?

- Programming can find the lastest data they used to continue their work.

12.